Project Documentation: Portable Weather Station

Video Link: https://youtu.be/jUAFbg8U60I

Course: 23CSE304 – Embedded System Case Study

Team Members:

- CB.SC.U4CSE23626 – J. Kanish Khan

- CB.SC.U4CSE23622 – Yaswanth

- CB.SC.U4CSE23635 – Prakyath

- CB.SC.U4CSE23619 – Hemanth

1. Introduction

Traditional weather monitoring systems are generally expensive, bulky, and lack real-time responsiveness for localized applications such as farms, households, or small communities.
To address this, our project presents a low-cost Smart Weather Station designed using the STM32F401 microcontroller that can continuously monitor environmental conditions and share data wirelessly.

2. Problem Statement

SMART WEATHER STATION: A LOW-COST EMBEDDED MONITORING SOLUTION

- Current Problem:
Most existing weather monitoring systems are costly and depend on large-scale infrastructure. They are not suitable for localized and real-time environmental tracking. Manual data collection and lack of automation make them inefficient for small-scale applications.

- Impact:
The absence of affordable and responsive weather systems affects preparedness against sudden weather changes such as unexpected rainfall. It impacts agriculture, outdoor operations, and small community safety.

3. Proposed Solution

We propose an embedded Smart Weather Station built using the STM32F401CCU6 microcontroller.
It gathers environmental data through various sensors and displays the information on an LCD while also transmitting it wirelessly via Bluetooth for remote access.

Key Features:

- Real-Time Monitoring:
Measures temperature, humidity, light intensity, and rainfall continuously.

- Automated Alerts:
Uses interrupts to detect sudden weather changes (e.g., rainfall).

- Wireless Data Sharing:
Transmits live weather data to a smartphone or PC using Bluetooth.

- User-Friendly Display:
Shows current readings clearly on a 16×2 LCD.

- Low-Cost and Scalable:
Affordable, compact, and adaptable for small or large setups (homes, farms, smart cities).

4. System Design

Block Diagram Overview

The STM32 microcontroller acts as the central processing unit.
It reads data from sensors and transmits it through UART for display and Bluetooth output.

Major Components:

- STM32F401CCU6 Microcontroller

- DHT22 Sensor – for Temperature and Humidity

- LDR Sensor – for Light Intensity

- Rain Sensor (Analog + Digital Output) – for rainfall detection

- 16×2 LCD with I²C (PCF8574 Backpack) – for on-screen data display

- HC-05 Bluetooth Module – for wireless communication

- LED Indicator and Buzzer – for alerts

5. Connection Diagram (Hardware Overview)

| Component | STM32 Pin | Function |
| --- | --- | --- |
| DHT22 | PA4 | Temperature & Humidity |
| LDR (Analog) | PA0 | Light Intensity |
| Rain Sensor (Analog) | PA1 | Rain Intensity |
| Rain Sensor (Digital) | PC13 | Rain Detection |
| LCD (I²C, PCF8574) | PB6 (SCL), PB7 (SDA) | Display Output |
| Bluetooth (HC-05) | TX/RX (USART2) | Wireless Communication |
| LED | PC13 | Status Indicator |
| Buzzer | PC14 | Alert Signal |

6. Program Workflow

Step 1: System Initialization

- Configure GPIO ports A, B, and C for input/output.

- Initialize ADC for LDR and rain sensors.

- Initialize UART for Bluetooth serial output.

Step 2: Startup Sequence

- Blink onboard LED to indicate power-on.

- Display initialization messages on LCD and via UART.

Step 3: Sensor Reading Loop

- Read DHT22 for temperature and humidity.

- Read LDR via ADC → Calculate light intensity.

- Read Rain Sensor (Analog) via ADC → Determine rainfall percentage.

- Read Rain Sensor (Digital) → Detect wet/dry status.

Step 4: Data Processing & Output

- Convert raw ADC values to percentages.

- Interpret environmental conditions (e.g., “BRIGHT / DARK”, “RAIN / NO RAIN”).

- Send formatted weather data through UART for display and Bluetooth monitoring.

Step 5: Alerts & Updates

- LED blinks every 3 seconds to show system activity.

- Continuously repeat sensor readings and updates.

7. Code Structure Overview

- main.c

  - Initializes GPIO, ADC, UART, and I²C.

  - Contains main sensor reading loop.

- dht22.c / dht22.h

  - Handles DHT22 data acquisition and temperature/humidity conversion.

- lcd_i2c.c / lcd_i2c.h

  - Controls LCD display using PCF8574 interface.

- adc_read.c

  - Reads analog values for LDR and rain sensor.

- bluetooth_uart.c

  - Transmits data via UART to connected Bluetooth devices.

8. Challenges Faced and Solutions

| Challenge | Cause | Solution Implemented |
| --- | --- | --- |
| Unstable DHT22 readings | Timing issues in signal pulse measurement | Implemented precise delay-based timing control |
| I²C LCD not displaying correctly | Wrong address or wiring issue | Corrected PCF8574 I²C address (0x4E) |
| Rain sensor analog fluctuations | ADC noise and sensitivity | Added averaging filter and calibration function |
| Bluetooth transmission lag | Baud rate mismatch | Configured UART at 9600 baud for stable transmission |

9. Results and Observations

- The system successfully measured and displayed:

  - Temperature and Humidity from DHT22

  - Light intensity from LDR

  - Rain detection from the rain sensor

- Wireless data transmission via Bluetooth was verified using a mobile serial terminal app.

- The LCD displayed continuous updates in real-time.

10. Applications

- Smart home automation systems

- Precision agriculture and irrigation control

- Localized weather monitoring for farms or schools

- Smart city environmental data collection

11. Conclusion

The Portable Weather Station provides a cost-effective, reliable, and scalable solution for real-time weather monitoring.
Using the STM32F401, we achieved accurate sensor integration, automated alerts, and seamless Bluetooth data sharing — all in a compact embedded system design.

12. Future Scope

- Add Wi-Fi connectivity (ESP8266/ESP32) for cloud data upload.

- Implement mobile app visualization.

- Include barometric pressure and wind sensors for advanced weather forecasting.

13. References

- STM32F4 Reference Manual – STMicroelectronics

- DHT22 Sensor Datasheet – Aosong Electronics

- PCF8574 I²C Backpack Documentation

- Embedded C programming examples from ST HAL libraries